BME688 Gas-Sensor AI Methodology
for the Multi-Sensor Fruit & Vegetable Freshness System

Agentic-AI implementation handoff document

# 1. Objective and Role of the BME688

Goal: Use the BME688 as an electronic-nose sensor inside a closed approximately 27.8 L container to detect changes in the volatile-gas fingerprint associated with produce deterioration. The BME688 should not be treated as a single “gas percentage” sensor and should not directly claim nutritional health.

The system has three sensing streams: 8 normal-LED RGB views from the Raspberry Pi camera, 8 synchronized UV-A views of the same fruit, and a BME688 gas/environment time-series. The BME688 branch should learn spoilage-related gas patterns and later feed a multimodal fusion model.

# 2. Core Design Principle

Fruit in sealed 27.8 L box
        ↓
BME688 heater-profile cycles
        ↓
Gas-response time series + T/RH/pressure
        ↓
BME688 classifier (BME AI-Studio and/or custom ML)
        ↓
Fresh / Early Spoilage / Rotten probabilities
        ↓
Fusion with RGB + UV-A visual features
        ↓
Final freshness / condition prediction

# 3. What to Measure

Do NOT build the dataset around a single value such as “Gas = 73%”. Preserve the complete response over time.

| Signal | Purpose | Keep in raw dataset? |
| --- | --- | --- |
| Timestamp | Time ordering / synchronization | Yes |
| Cycle / sample index | Identify position in gas response sequence | Yes |
| Heater-profile step | Preserve multi-step gas-response information | Yes |
| Gas resistance / gas signal | Primary gas fingerprint | Yes |
| Temperature | Environmental context and confounder check | Yes |
| Relative humidity | Environmental context and confounder check | Yes |
| Pressure | Environmental context | Yes |
| Sensor/device ID | Reproducibility / hardware tracking | Yes |
| Fruit/specimen metadata | Ground truth association | Yes |

# 4. Physical Experimental Setup

Use the same sealed approximately 27.8 L box geometry for training and evaluation whenever possible.

Keep BME688 position fixed. Do not move it between experiments.

Keep fruit placement fixed or document its position.

Keep the sensor-to-produce distance as consistent as practical.

Use the same hardware, wiring, power configuration, and heater-profile/duty-cycle configuration for data intended for the same model.

Record environmental conditions because temperature and humidity can affect sensor responses and can become accidental shortcuts for the model.

Do not expose the sensor to uncontrolled sprays, cleaning chemicals, perfume, smoke, or other strong VOC sources during experiments.

Bosch reference: BME AI-Studio documentation — use it for the official BME688 data-recording, specimen, training, and deployment workflow.

# 5. Experimental Data Collection Protocol

Start with an empty-box baseline. Record the BME688 response with the same operating configuration used for produce experiments. Repeat baseline measurements across multiple days if possible.

Place one known specimen in the box and close the container.

Allow a consistent stabilization/measurement period before comparing results across specimens. Do not vary the measurement duration arbitrarily.

Record the BME688 continuously through repeated heater-profile cycles. Preserve the raw time series.

At the same specimen/time point, capture the 8 RGB views and 8 UV-A views so all three sensing streams share the same specimen ID.

Assign a ground-truth condition based on the physical state of the produce, not merely on the storage day.

Repeat over multiple days of storage and across many different specimens of each target produce type.

Repeat at different reasonable quantities/masses so the model does not learn “more fruit = rotten”.

Repeat under different realistic temperatures/humidity conditions so the model is tested against environmental variation.

# 6. Recommended BME688 Classes

Recommended first-stage classification task:

Class 0 = Empty / Background
Class 1 = Fresh
Class 2 = Early Spoilage
Class 3 = Rotten

Use these as the initial BME688 targets. Ripeness is a separate concept and should not automatically be mapped to spoilage.

A ripe banana is not automatically rotten.

A bruised fruit is not automatically rotten.

Storage day is not the label; physical condition is the label.

# 7. The Most Important Dataset Unit: SPECIMEN_ID

The individual fruit or vegetable specimen should be the fundamental experimental unit. The 16 camera images and the BME688 recording for that specimen must all share one identifier.

SPECIMEN_ID = BAN_027

RGB: 8 synchronized angles
UV-A: 8 synchronized angles
BME688: full time-series
Ground truth: Early Spoilage

# 8. Synchronize RGB, UV-A and BME688

For each specimen, capture corresponding views:

RGB_000 ↔ UV_000
RGB_045 ↔ UV_045
RGB_090 ↔ UV_090
RGB_135 ↔ UV_135
RGB_180 ↔ UV_180
RGB_225 ↔ UV_225
RGB_270 ↔ UV_270
RGB_315 ↔ UV_315

The BME688 does not have “angles”. It should instead be recorded as a time-series associated with the same SPECIMEN_ID and experiment session.

# 9. Recommended Storage Structure

dataset/
├── metadata.csv
├── BAN_001/
│   ├── rgb/
│   │   ├── 000.jpg
│   │   ├── 045.jpg
│   │   └── ...
│   ├── uv/
│   │   ├── 000.jpg
│   │   ├── 045.jpg
│   │   └── ...
│   └── bme688/
│       ├── raw.csv
│       └── summary.json
└── ...

# 10. Required Metadata Schema

| Field | Example | Why it matters |
| --- | --- | --- |
| Specimen_ID | BAN_027 | Joins RGB, UV-A, gas data and labels |
| Fruit_type | Banana | Condition can be fruit-dependent |
| Variety | Cavendish (if known) | Controls biological variation |
| Weight_g | 132 | Controls gas-load effects |
| Storage_day | 4 | Time context; not the ground truth itself |
| Quantity | 3 fruits | Prevents quantity leakage |
| Box_volume_L | 27.8 | Experimental constant |
| Sensor_ID | BME688_01 | Tracks hardware differences |
| Heater_profile | HP_xxx | Critical for reproducibility |
| Duty_cycle | RDC_xxx | Critical for reproducibility |
| Temperature | 28.2 C | Environmental confounder |
| Humidity | 71 %RH | Environmental confounder |
| Pressure | 1007 hPa | Environmental context |
| Start_time | YYYY-MM-DD HH:MM:SS | Temporal synchronization |
| Duration_min | 30 | Measurement consistency |
| Ground_truth | Early Spoilage | Primary target |
| Visible_mold | No | Auxiliary physical label |
| Bruise | Minor | Auxiliary physical label |
| RGB_path | ... | Links visual data |
| UV_path | ... | Links UV-A data |
| BME_path | ... | Links gas data |
| Notes | Mild brown spots | Human observation / anomalies |

# 11. Ground Truth: How to Label the Fruit

Do not let the AI decide its own training labels. The dataset needs a human/experimental reference label. Use a consistent rubric.

| Condition | Typical meaning | BME target |
| --- | --- | --- |
| Fresh | No meaningful spoilage signs; acceptable produce | Fresh |
| Ripe / Overripe but not spoiled | Maturity state without spoilage; keep distinct from spoilage | Fresh unless actual spoilage is present |
| Early Spoilage | Initial deterioration/decay signs; not yet severely spoiled | Early Spoilage |
| Rotten / Spoiled | Clear spoilage/decay and unacceptable condition | Rotten |

# 12. Data Quantity Strategy

For the BME688, variety and independence matter more than collecting many repeated readings from one fruit. A single fruit measured for 100 cycles is still one biological specimen. Build the dataset around many independent specimens and multiple storage conditions.

Use multiple specimens per fruit/vegetable type.

Follow some specimens longitudinally across multiple storage days.

Keep some separate specimens reserved for final testing.

Do not let all images/time points from one specimen appear in both training and test sets; this causes leakage.

# 13. Training the BME688 Model: First Path

Use Bosch BME AI-Studio as the first reference implementation. The intended workflow is:

Record raw BME688 data
        ↓
Define specimens / sessions
        ↓
Assign labels
        ↓
Train classifier
        ↓
Evaluate classification performance
        ↓
Export / deploy the trained configuration as appropriate

This gives you a sensor-native baseline before building your own custom ML pipeline.

# 14. Custom ML Benchmark After BME AI-Studio

Once the raw dataset is stable, train independent models so you can compare the Bosch workflow with conventional machine learning.

Random Forest — strong baseline for tabular engineered sensor features; no feature scaling required.

XGBoost — strong tabular baseline and a good candidate for the final fusion model.

SVM — useful comparison for smaller datasets.

MLP — optional neural-network baseline on engineered/aggregated features.

1D CNN or LSTM/GRU — later research options if you want to learn directly from gas-response sequences.

# 15. Feature Extraction for Custom ML

Preserve raw data first. Then create a derived feature table for model experiments.

Per measurement/session, possible derived features:
- mean gas resistance
- median gas resistance
- minimum / maximum
- standard deviation
- peak response
- response slope
- recovery slope
- area under response curve
- heater-step response values
- temperature statistics
- humidity statistics
- pressure statistics
- temporal change between cycles

Do not replace the raw dataset with derived features. Keep both.

# 16. Normalization / Scaling Rule

Important: Do not blindly apply min-max scaling to the BME688 data.

Tree-based models such as Random Forest and XGBoost generally do not require feature scaling.

If a neural network requires scaling, use a clearly documented training-set transformation (for example standardization/z-score) and save that transformation with the model.

Never destroy or overwrite the raw BME688 values.

# 17. Prevent the Model from Learning the Wrong Thing

This is one of the highest-risk parts of the project. The gas model can accidentally learn a shortcut instead of spoilage.

Do not correlate one fruit type with one condition only. Otherwise the model can learn fruit identity instead of spoilage.

Do not correlate quantity with condition. Otherwise more produce can look like “rotten”.

Do not correlate day number with condition. Otherwise the model can learn storage time instead of gas signature.

Vary temperature and humidity within realistic ranges.

Include empty-box/control sessions.

Use different specimens for test data.

# 18. How to Make a Real-Time Prediction

At inference time, do not make a final decision from one instantaneous BME688 reading. Collect a consistent window of gas-response cycles and classify the resulting sequence/session.

New specimen
   ↓
Collect BME688 measurement window
   ↓
Apply the exact same preprocessing / feature extraction used during training
   ↓
BME688 classifier
   ↓
Fresh probability
Early-Spoilage probability
Rotten probability
   ↓
Aggregate over the measurement window
   ↓
Stable BME688 prediction

For example, if successive cycles fluctuate slightly, use an agreed aggregation method such as probability averaging over the latest window rather than declaring “rotten” from one noisy cycle.

# 19. Example BME688 Prediction Output

SPECIMEN: BAN_027

Fresh            0.12
Early Spoilage   0.76
Rotten           0.12

BME688 result: EARLY SPOILAGE
Gas-model confidence: 0.76

The confidence/probability values should come from the trained model and should not be presented as a physical gas concentration.

# 20. Fusion with RGB and UV-A

The BME688 result should be one evidence source, not the entire final decision. The final model should combine the three modalities.

RGB 8-view encoder
        ↓
RGB visual features

UV-A 8-view encoder
        ↓
UV-A features

BME688 time-series model
        ↓
Gas features
        ↓
        ┌────────────────────┐
        │  Fusion model      │
        │  XGBoost / MLP     │
        └─────────┬──────────┘
                  ↓
Final freshness / condition probabilities

# 21. Recommended Fusion Inputs

Fruit type predicted by the RGB branch (or supplied if known).

RGB 360-degree visual representation.

UV-A 360-degree representation / anomaly features.

BME688 gas-response features.

Temperature and humidity context.

Optional specimen-level metadata such as weight/quantity if these are known and controlled.

# 22. Experimental Comparison You Should Report

Your report should demonstrate whether the gas sensor actually adds value.

| Experiment | Inputs | Purpose | Report |
| --- | --- | --- | --- |
| RGB-only | 8 RGB views | Camera baseline | Accuracy, precision, recall, F1, confusion matrix |
| UV-only | 8 UV-A views | UV contribution | Same metrics |
| RGB + UV-A | 16 images | Multispectral visual baseline | Same metrics |
| BME688-only | Gas sequence | Electronic-nose baseline | Same metrics |
| RGB + BME688 | RGB + gas | Does gas add value? | Same metrics |
| RGB + UV-A + BME688 | All modalities | Final proposed system | Same metrics + per-class results |

# 23. What to Put in the Final Results / Thesis

Dataset composition by fruit type and condition.

Number of independent specimens, measurement sessions, and time points.

Environmental ranges: temperature and humidity.

BME688 heater profile / duty cycle used.

Example raw gas-response curves for Fresh vs Early Spoilage vs Rotten.

BME AI-Studio model performance.

Custom Random Forest / XGBoost / other model comparison.

Confusion matrix and F1 score, not accuracy alone.

Ablation comparison showing RGB, UV-A, BME688 and full fusion.

Failure cases and conditions where the gas model is uncertain.

# 24. Example Experiment Record

SPECIMEN_ID: TOM_014
Fruit: Tomato
Weight: 118 g
Quantity: 1
Storage day: 4
Box volume: 27.8 L
Temperature: 27.8 C
RH: 69 %
BME duration: 30 min
Heater profile: HP_xxx
Duty cycle: RDC_xxx

RGB: 8 images saved
UV-A: 8 images saved
BME688: raw.csv saved

Ground truth: Early Spoilage
Visible mold: No
Bruising: Minor
Notes: Slight softening

# 25. Example Dataset Record (CSV-style)

SPECIMEN_ID,Fruit,Day,Weight_g,Quantity,Temp_C,RH_pct,Ground_Truth,RGB_Path,UV_Path,BME_Path
BAN_027,Banana,4,132,1,28.2,71,Early_Spoilage,BAN_027/rgb,BAN_027/uv,BAN_027/bme688/raw.csv

# 26. Implementation Rules for the Agentic AI

Preserve the raw BME688 readings and metadata exactly as collected.

Build the data pipeline around SPECIMEN_ID and synchronized RGB/UV-A/BME688 sessions.

Implement Empty, Fresh, Early Spoilage and Rotten as the first BME688 classes.

Train a BME AI-Studio baseline before custom ML.

Train Random Forest and XGBoost on engineered gas-response features as conventional baselines.

Do not use min-max normalization as a default step.

Prevent specimen leakage between train/validation/test splits.

Evaluate BME688-only before doing fusion.

Use the BME688 result as probabilistic evidence for the final RGB + UV-A + BME688 fusion model.

Aggregate predictions across a measurement window instead of reacting to one instantaneous sensor sample.

Log the final prediction together with model probabilities, sensor conditions, specimen ID and measurement time for every real-world test.

# 27. Real-Time System Log Format

timestamp
specimen_id
fruit_type
measurement_window_start
measurement_window_end
temperature
humidity
BME_fresh_probability
BME_early_spoilage_probability
BME_rotten_probability
RGB_quality_score
UV_anomaly_score
final_fresh_probability
final_warning_probability
final_rotten_probability
final_class
model_version

# 28. Safety and Scientific Limitations

The BME688 does not directly measure “rottenness”; it measures gas-sensor responses that must be learned and empirically related to your ground truth.

Do not claim that the BME688 identifies a specific chemical compound unless your experimental setup actually supports that claim.

Do not claim food safety solely from the model output. The system is a non-destructive freshness/spoilage prediction system.

UV-A is a separate optical modality; use appropriate shielding and safe operating procedures.

# 29. Final Recommended Development Sequence

Phase 1  → Empty-box + Fresh/Rotten pilot recordings
Phase 2  → Longitudinal measurements across storage days
Phase 3  → Multi-specimen dataset across all target produce
Phase 4  → BME AI-Studio baseline
Phase 5  → RF / XGBoost gas-model benchmark
Phase 6  → RGB and UV-A feature extraction
Phase 7  → RGB + UV-A visual fusion
Phase 8  → RGB + UV-A + BME688 final fusion
Phase 9  → Held-out specimen evaluation
Phase 10 → Raspberry Pi 5 deployment / optimization

# 30. Official / Key References

Bosch Sensortec — BME AI-Studio documentation: https://www.bosch-sensortec.com/software/bme/docs/

Bosch Sensortec — BME AI-Studio: Record Data: https://www.bosch-sensortec.com/software/bme/docs/process-steps/record-data.html

Bosch Sensortec — BME AI-Studio: Train Algorithms: https://www.bosch-sensortec.com/software/bme/docs/process-steps/train-algorithms.html

Bosch Sensortec — BME AI-Studio: Import Data: https://www.bosch-sensortec.com/software/bme/docs/process-steps/import-data.html

End of implementation handoff document